Image Segmentation Techniques

| Main Segmentation Technique | Sub-Technique / Model | Lung Cancer | Breast Cancer | Brain Tumor |
| --- | --- | --- | --- | --- |
| Traditional Image Segmentation | Thresholding (Otsu) | Lung field extraction from CT/X-ray | Mass boundary in mammograms | Gray/white matter separation in MRI |
|  | Region Growing | Nodule detection | Mass segmentation in ultrasound | Tumor region expansion from seed point |
|  | Watershed | Nodule separation (touching structures) | Nuclei separation in histopathology | Tumor boundary refinement |
|  | K-Means Clustering | Tissue type separation in CT | Lesion clustering in ultrasound | Tumor/edema clustering in MRI |
|  | Fuzzy C-Means | — | Dense tissue segmentation | Most common for brain MRI (soft boundaries) |
|  | Active Contour (Snake) | Nodule boundary refinement | Mass boundary detection | Tumor contour evolution |
|  | Graph Cut | Lung parenchyma segmentation | — | Tumor/background separation |
|  | Level Set Method | Nodule shape modeling | — | Tumor boundary evolution in MRI |
|  | Atlas-Based Segmentation | — | — | Standard brain atlas registration |
| Semantic Segmentation | FCN | Lung region segmentation (baseline) | Mass region segmentation | Tumor region segmentation |
|  | U-Net | Lung field/nodule segmentation (most used) | Mass/lesion segmentation | BraTS benchmark standard |
|  | U-Net++ | Fine-grained nodule boundaries | Lesion boundary refinement | Fine tumor sub-region detection |
|  | Attention U-Net | Nodule-focused segmentation | Mass region with reduced false positives | Tumor sub-region focus (core/edema) |
|  | ResUNet / ResUNet++ | Deep feature nodule segmentation | Lesion segmentation | Tumor segmentation with residual learning |
|  | SegNet | Lightweight lung segmentation | — | Tumor segmentation (memory-efficient) |
|  | V-Net (3D) | 3D lung nodule volume segmentation | — | 3D tumor volume segmentation |
|  | DeepLabV3+ | Lung nodule segmentation | — | Multi-scale tumor context |
|  | PSPNet | Multi-scale nodule detection | — | Multi-scale tumor region parsing |
|  | nnU-Net | Lung tumor (self-configuring) | Breast lesion (self-configuring) | Brain tumor (self-configuring, top BraTS performer) |
| Instance Segmentation | Mask R-CNN | Individual nodule detection (multiple nodules) | Individual nuclei segmentation in histopathology | — |
|  | Cascade Mask R-CNN | Multi-nodule detection | Cell detection in histopathology | — |
|  | YOLACT | Real-time nodule instance segmentation | Blood/cell detection | — |
|  | SOLO / SOLOv2 | Nodule instance segmentation | Histology cell instances | — |
| Panoptic Segmentation | Panoptic FPN | Lung + nodule combined segmentation | — | Organ + tumor combined |
|  | Panoptic-DeepLab | Multi-structure lung CT analysis | — | Whole-brain organ + tumor analysis |
| Transformer-Based Segmentation | TransUNet | Lung tumor segmentation | — | Brain tumor segmentation |
|  | Swin-UNet | Lung CT segmentation | — | Brain tumor (BraTS) |
|  | UNETR | 3D lung CT volumes | — | 3D brain MRI volumes |
|  | Medical SAM / MedSAM | Interactive nodule annotation | Interactive lesion annotation | Interactive tumor annotation |
| Weakly / Semi-Supervised Segmentation | Multiple Instance Learning (MIL) | — | Whole-slide image classification (slide-level labels only) | — |
|  | CAM / Grad-CAM based | Nodule localization from classifier | Lesion localization from classifier | Tumor localization from classifier |